Problem 1

Show that as the sampling period Ts approaches zero, the formula for the discrete-time Fourier transform given in approaches the formula for the Fourier transform.

Answer:

GD(f)=n=-∞∞ gn2Bexp⁡-jπnfB

The sampling period Ts=1/(2 B). So, it can be rewritten.

GD(f)=n=∞∞ gnTsexp⁡-j2πnTsf

As Ts approaches zero, the discrete time nTs, approaches continuous time t. Moreover, the summation over n approaches the equation below.

-∞∞ g(t)exp⁡(-j2πtf)dt

GD(f) approaches the continuous FT G(f). So, the formula for the DFT GD(f) approaches the formula for the FT:

G(f)=-∞∞ g(t)exp⁡(-j2πtf)dt

as the sampling period Ts approaches zero.

Problem 2

g(t)=k  gkTssinc(2πBt-kπ)

the interpolation formula can be written like this

g(t)=k=-∞∞ gk2B12B-BB exp⁡2πjft-k2Bdf

Prove the equation below.

12B-BB exp⁡2πjft-k2Bdf =sinc(2πBt-kπ)

Answer:

12B-BB  exp⁡j2πft-k2Bdf=12B⋅1j2π(t-k/2B)⋅exp⁡j2πft-k2Bf=-BB =1j4πB(t-k/2B)⋅[exp⁡jπ(2Bt-k)-exp⁡(-jπ(2Bt-k))] =sin⁡(π(2Bt-k))π(2Bt-k) =sinc(2πWt-kπ)

Problem 3

Specify the Nyquist rate and the Nyquist interval for each of the following signals:
(a) g(t)=sinc3(200πt)
(b) g(t)=sinc3(200πt)+sinc4(200πt)

Answer

Sinc is rectangular with 2B BW in FT domain. Sinc2  is the convolution of two rectangular signal that extends the BW to 4BW. Sinc3 extends the BW 3 times and Sinc4  extends the BW 4 times.

a)

Sinc BW is 100Hz.

Sinc2  BW is 200 Hz.

Sinc3 BW is 300 Hz. So, Nyquist rate is 600Hz and Nyquist interval is 1.66667ms

b)

Sinc4 BW is 400 Hz. And this is the highest signal frequency in (b). S0, Nyquist rate is 800 Hz, and Nyquist interval is 1.25 ms.

Problem 4

Consider uniform sampling of the sinusoidal wave

g(t)=cos⁡(πt)

Determine the Fourier transform of the sampled waveform for the following sampling period:
(a) Ts=0.25 s
(b) Ts=1 s
(c) TS=1.5 s

Answer

g(t)=cos⁡(πt)

This is a cosine wave with angular frequency:

ω=π⇒f=ω2π=12 Hz

So, its Fourier Transform is:

G(f)=12[δ(f-0.5)+δ(f+0.5)]

If g(t) is sampled at period Ts, the sampled signal is:

gs(t)=g(t)⋅n=-∞∞ δt-nTs

In the frequency domain, this gives:

Gs(f)=k=-∞∞ 1TsGf-kfs,          fs=1Ts

So:
The spectrum becomes periodic with spacing fs, and each replica is scaled by 1Ts.

Case (a)

Ts=0.25 s⇒fs=4 Hz

- No aliasing occurs, because fs>2fmax=1 Hz

Replicas will be centered at:

f=±0.5,±(0.5+4),±(0.5+8),…

So:

Gs(f)=12Tsk=-∞∞  δf-0.5-kfs+δf+0.5-kfs

Plugging in Ts=0.25 :

Gs(f)=2⋅k=-∞∞ [δ(f-(0.5+4k))+δ(f+(0.5-4k))]

Case (b) Ts=1 s⇒fs=1 Hz

- fs=2fmax → Critical Nyquist rate

Replicas are at:

f=±0.5,±1.5,±2.5,…

No aliasing, but spectrum just touches at the boundary.

Gs(f)=k=-∞∞ δf-0.5-kfs+δf+0.5-kfs

Case (c) Ts=1.5 s⇒fs=23 Hz

- Now: fs<2fmax→ Aliasing occurs

Aliased frequency:

falias =f0-kfs⇒0.5-k⋅23

Try k=1 :

|0.5-0.6667|=0.1667 Hz

So the peaks are now incorrectly shifted due to aliasing.

Problem 5

Derive the slope characteristics of  μ -law and A -law and plot for values of μ=1000,100,10,0  and A=1000,87.6,10,1

y=1ln⁡(1+μ)ln⁡1+μmmp 0≤mmp≤1

y=A1+ln⁡Ammp0≤mmp≤1A11+ln⁡A1+ln⁡Ammp1A≤mmp≤1

Solution
(a) The logarithmic law is defined by

y=1ln⁡(1+μ)ln⁡1+μmmp 0≤mmp≤1

Therefore, differentiation with respect to m= mmp yields

dyd|m|=1ln⁡(1+μ)⋅μ1+μ|m|

(b) The A-law is defined by (see Eq. (5.25):

y=A1+ln⁡Ammp0≤mmp≤1A11+ln⁡A1+ln⁡Ammp1A≤mmp≤1

Hence, differentiation with respect to |m| yields

dyd|m|=A1+ln⁡A,0≤|m|≤1AA|m|(1+ln⁡A),1A≤|m|≤1

Problem 6

Consider a sinusoidal wave of frequency fm and amplitude Am, applied to a delta modulator of step size Δ. Show that slope-overload distortion will occur if

Am>Δ2πfmTs

where Ts is the sampling period. What is the maximum power that may be transmitted without slope-overload distortion?

Answer:

The modulating wave is

m(t)=Amcos⁡2πfmt

The slope of m(t) is given by

dm(t)dt=-2πfmAmsin⁡2πfmt

The maximum slope of m(t) is therefore equal to 2πfmrAm.
The maximum average slope of the approximating signal ma(f) produced by the delta modulator is E/Ts, whereE is the step size and Ts is the sampling period. The limiting value of Am is therefore given by

2πfmAm>ETs or Am>E2πfmTs

Assuming a load of 1 ohm , the transmitted Average power is Am2/2. Therefore, the maximum power that nay be transmitted without slope-overload distortion is equal to E2/8π2fm2Ts2.

Problem 7

Consider a delta modulation (DM) system used to transmit a voice signal, which is uniformly sampled at the rate of 64 kHz . Assume the following specifications:

Voice signal bandwidth  =3.1kHz Maximum signal amplitude  =10 volts

(a) To avoid slope overload distortion, what is the minimum permissible value of the step size Δ used in the system?
(b) Determine the average power of granular noise.
(c) Determine the minimum-channel bandwidth needed to transmit the DM encoded data.

Answer:

Sampling rate =64kHz
Voice signal bandwidth =W=3.1kHz
Maximum signal amplitude Amax=10 volts
(a) To avoid slope overload, we must satisfy the following requirement

Amax<Δ2πWTs

Solving for the step size Δ, we write

Δ>12πWTsAmax=fs2πWAmax

Substituting the given values into yields

Δ>642π×3.1×10

or

Δ>0.33 volts
Provided that the step size Δ is 0.33 volt, then slope-overload distortion is avoided.

(b)

Let ∈(t) denote the granular noise, viewed as a function of time t. The average power of granular noise (analogous to quantization noise in PCM), is defined by

Pg =2Δ-Δ/2Δ/2  ϵ2d∈ =2Δϵ33ϵ=-Δ/2Δ/2 =Δ23

With Δ set at 0.33 volt, the average power of granular noise is therefore 0.03 watts (assuming that the power is calculated for a load of 1 ohm ).

(c) The minimum channel bandwidth needed to transmit the DM encoded signal is the inverse of the sampling rate 64 kHz if RZ.

For NRZ it’s 32KHz.

Problem 8

Consider a compact disc that uses pulse-code modulation to record audio signals whose bandwidth W=15kHz. Specifications of the modulator include the following:
Quantization: uniform with 512 levels
Encoding: binary
Determine (a) the Nyquist rate, and (b) the minimum permissible bit rate.

Answer

(a) The Nyquist rate is 2W=30kHz.
(b) To accommodate 512 quantization levels, we require a binary code with B bits, which would have to satisfy the following requirement:

2B=512

Hence, B=9. The sampling period Ts=1/30 milliseconds must be divided into 9 bits. The minimum sampling rate is therefore

30×9 =270 kilobits / second  =0.27 megabits / second

Problem 9

This problem addresses the digitization of a television signal using pulse-code modulation. The signal bandwidth is 4.5 MHz . Specifications of the modulator include the following:
Sampling:  15% in excess of the Nyquist rate
Quantization: uniform with 1024 levels
Encoding: binary
Determine (a) the Nyquist rate, and (b) the minimum permissible bit rate.

(a) We are given

- Video bandwidth =4.5MHz

- Sampling rate =15% in excess of the Nyquist rate

- Uniform quantization using 1024 levels

- Binary encoding
(b) The Nyquist rate is 2×4.5=9MHz. Actual sampling rate =9×1.15=10.35MHz
The sampling period is therefore

Ts=110.35μ s

This sampling rate must be divided into B bits, where

2B=1024

Hence, B=10. The bit duration is therefore

Ts10=1103.5μ s

The permissible bit rate is therefore 103.5 megabits/s.

Problem 10

This shows a PCM signal in which the amplitude levels of +1 volt and -1 volt are used to represent binary symbols 1 and 0 , respectively. The code word used consists of three bits. Find the sampled version of an analog signal from which this PCM signal is derived.

Answer

The transmitted code words, representing the PCM waveform

| t/Tb | code |
| --- | --- |
| 1 | 001 |
| 2 | 010 |
| 3 | 011 |
| 4 | 100 |
| 5 | 101 |
| 6 | 110 |

Accordingly, the sampled analog signal from which these code words are derived is shown below